Completing This Checklist:

Please follow and complete this checklist in the order below. Vehicle units being certified are to be weighed and inspected in an operational ready configuration to ensure accurate S10 calculations. There are particulars in this checklist that have reference numbers e.g. (B7). The data from these particulars will be used in other areas of the checklists to complete the necessary calculations as part of the S10 certification. Please follow the references. For further information regarding the use of this checklist and calculations please email TMR. Refer to Appendix 1 of the S10 code for a glossary of terms referenced in this checklist.

Part A - Owner/Supplier & Vehicle Details

## Vehicle Owner/Supplier Details

| Name: | Company/business: |  |
| --- | --- | --- |
| Address: |  | Post code: |

## Vehicle Details

| Vehicle Details |  |  |  |  |
| --- | --- | --- | --- | --- |
| Vehicle Make: | Vehicle Model: |  |  | Month and Year of Manufacture: |
| VIN: |  | Vehicle Chassis Number (if applicable): | Engine Number: |  |

Part B – Vehicle Dimensions

## Vehicle Layout & Dimensions

| Vehicle Manufacturer's Ratings |  |  |  |  |
| --- | --- | --- | --- | --- |
| (B1) Gross Vehicle Mass (GVM): |  | (B2) Gross Combination Mass (GCM): |  |  |
| kg |  | kg |  |  |
| (B3) Front Axle Load Rating: |  | (B4) Rear Axle Load Rating: |  |  |
| kg |  | kg |  |  |
| Vehicle Dimensions |  |  |  |  |
| (B5) Vehicle Wheelbase: |  |  |  |  |
| m |  |  |  |  |

## Vehicle Components

| Component Specifications |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Axles |  |  |  |  |  |  |  |  |  |
| Front Axle Group |  |  |  |  |  |  |  |  |  |
| Make: |  | Model: |  |  | (B6) Axle Manufacturer's Load Rating1: |  |  |  | Number of Axles: |
|  |  |  |  |  | kg |  |  |  |  |
| Rear Axle Group |  |  |  |  |  |  |  |  |  |
| Make: |  | Model: |  |  | (B7) Axle Manufacturer's Load Rating2: |  |  |  | Number of Axles: |
|  |  |  |  |  | kg |  |  |  |  |
| Suspension |  |  |  |  |  |  |  |  |  |
| Front Axle Group |  |  |  |  |  |  |  |  |  |
| Make: |  | Model: |  |  | (B8) Suspension Manufacturer's Load Rating1: |  |  |  |  |
|  |  |  |  |  | kg |  |  |  |  |
|  |  |  |  |  | Load Sharing: |  |  | Y ​ ☐ ​ N ☐ ​ N/A ☐ |  |
| Rear Axle Group |  |  |  |  |  |  |  |  |  |
| Make: |  | Model: |  |  | (B9) Suspension Manufacturer's Load Rating2: |  |  |  |  |
|  |  |  |  |  | kg |  |  |  |  |
| Tyres |  |  |  |  |  |  |  |  |  |
| Front |  |  |  |  |  |  |  |  |  |
| (B10) Tyre Size: |  |  | (B11) Load Rating/Capacity Per Tyre3: |  |  | (B12) Total Axle Group Tyre Load Capacity1: |  |  |  |
|  |  |  | kg |  |  | kg |  |  |  |
| Rear |  |  |  |  |  |  |  |  |  |
| (B13) Tyre Size: |  |  | (B14) Load Rating/Capacity Per Tyre4: |  |  | (B15) Total Axle Group Tyre Load Capacity2: |  |  |  |
|  |  |  | kg |  |  | kg |  |  |  |
| - NOTE:- The limiting factor is the lowest value of B6, B8 and B12. - NOTE:- The limiting factor is the lowest value of B7, B9 and B15. - Minimum single tyre load rating. - Minimum dual tyre load rating. If more than one load rating is fitted, list all different load ratings. |  |  |  |  |  |  |  |  |  |
| Coupling |  |  |  |  |  |  |  |  |  |
| Fifth Wheel/Turntable |  |  |  |  |  |  |  |  |  |
| Make: |  |  |  | Model: |  |  | (B16) D-value: |  |  |
|  |  |  |  |  |  |  | kN |  |  |
| (B17) Fifth Wheel Offset5: | m |  |  |  |  |  |  |  |  |
| - Fifth wheel lead is a positive value. Fifth wheel lag is a negative value. |  |  |  |  |  |  |  |  |  |

## Assessment – Vehicle Design

| Assessment – Vehicle Design Check Yes, No, N/A as applicable: |  |  | Yes |  | No |  | N/A |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| If the answer is 'No' to any of the assessments then S10 certification MUST NOT proceed. |  |  |  |  |  |  |  |  |
| 1 | Are all the drive axles fitted with dual tyres? |  |  | ☐ |  | ☐ |  | ☐ |
| 2 | Is the vehicle's steering configuration either single steer or twin-steer with load sharing suspension? |  |  | ☐ |  | ☐ |  | ☐ |
| 3 | Are the vehicle dimensions unchanged, and do they continue to comply with relevant ADRs and regulations? |  |  | ☐ |  | ☐ |  | ☐ |
| 4 | Is the vehicle fitted with either: |  |  |  |  |  |  |  |
|  | - a single drive axle for vehicles with a single rear axle; or |  |  | ☐ |  | ☐ |  | ☐ |
|  | - at least tandem drive axles for vehicle with multiple rear axles? |  |  | ☐ |  | ☐ |  | ☐ |
| 5 | Are the load ratings (B16), for fitted couplings, greater than or equal to the following? |  |  |  |  |  |  |  |
|  | - For Type 2 road train hauling units | 160kN |  | ☐ |  | ☐ |  | ☐ |
|  | - For Type 1 road train hauling units | 140kN |  | ☐ |  | ☐ |  | ☐ |
|  | - For other units | 123kN |  | ☐ |  | ☐ |  | ☐ |

Part C – Vehicle Equipment & Unladen Mass

Vehicle Equipment

Vehicle Equipment

The vehicle must be ready for service, with all accessories and equipment fitted before the vehicle is weighed. List all equipment that is fitted to the vehicle at the time of weighing to determine the unladen mass. This includes all fuel, water and fluid reservoirs, accessories (e.g. bulbar, lights, fridge, TV etc.) and readily removable equipment carried on or in the vehicle during operational use (e.g. spare tyre(s), toolbox(es), OH&S equipment etc.). It is recommended that detailed photographs are taken of the vehicle at the time of weighing that capture all the equipment fitted. Attach all photos in the space provided at the end of Part C.

Vehicle Equipment List:

Vehicle Unladen Mass

| Weigh Vehicle |  |
| --- | --- |
| Effluent tanks, if fitted, must be empty when the vehicle is weighed. All other fluid reservoirs must be filled to capacity when the vehicle is weighed. Weigh vehicle at a registered public weighbridge to determine the Total Vehicle Unladen Mass. Attach a copy of the weighbridge ticket in the space provided below. An additional mass of 40kg for driver/operator personal items is to be included in the calculations for Total Vehicle Unladen Mass (C3). 20kg is to be added to both C1 and C2 respectively. If required, an additional toolbox that will be fitted after the vehicle weigh-in will need to be applied as a simulated mass of 100kg. Refer to the Vehicle Checklist Drawings. |  |
| (C1) Front Axle Group Unladen Mass (including additional 20kg): | kg |
| (C2) Rear Axle Group Unladen Mass (including additional 20kg): | kg |
| (C3) Total Vehicle Unladen Mass: | kg |
| Vehicle Weighbridge Ticket – Attach a copy of the ticket below: |  |
| Photos of the vehicle at the time of weighing: |  |

Part D – Vehicle Layout & S10 Laden Mass

Vehicle Layout & S10 Laden Mass

| Calculate S10 Laden Mass |  |
| --- | --- |
| Accredited Approved Vehicle Examiners (AVE) are encouraged to complete S10 calculations using the Vehicle Drawings. AVEs may use their own drawings to complete S10 calculations provided that all assumptions and methodology are clear and concise. Apply 15,000kg vertically through the centre of the fifth wheel coupling to simulate loading from a laden semitrailer. Complete the calculations and record the results for the weight distribution and Vehicle S10 Laden Mass below. |  |
| Imposed Load from Semitrailer |  |
| (D1) Total Vehicle Imposed Load: | kg |
| Weight Distribution |  |
| (D2) Forward Axle Mass (FAM)*: | kg |
| (D3) Rearward Axle Mass (RAM)*: | kg |
| *Note:- As a check the sum of FAM and RAM from the imposed load should equal 15,000kg |  |
| Front and Rear Axle S10 Laden Masses |  |
| (D4) Front Axle Group S10 Laden Mass: (C1 + D2) | kg |
| (D5) Rear Axle Group S10 Laden Mass: (C2 + D3) | kg |
| (D6) Vehicle S10 Laden Mass: (D4 + D5) | kg |
| S10 Conditional Mass Limits (Refer to Schedule 1, Section 1 of the Queensland Class 3 Livestock Loading Exemption Notice) |  |
| (D7) Front Axle Group S10 Conditional Mass Limit: | kg |
| (D8) Total Laden S10 Conditional Mass Limit: | kg |

Assessment – S10 Laden Mass

| Assessment – S10 Laden Mass Check Yes or No as applicable: |  | Yes | No |
| --- | --- | --- | --- |
| If the answer is 'No' to any of the assessments then S10 certification MUST NOT proceed. |  |  |  |
| 1 | Is the Front Axle Group S10 Laden Mass (D4) less than or equal to the vehicle manufacturer’s Front Axle Load Rating (B3)? | ☐ | ☐ |
| 2 | Is the Front Axle Group S10 Laden Mass (D4) less than, or equal to the Axle Manufacturer's Load Rating (B6)? | ☐ | ☐ |
| 3 | Is the Front Axle Group S10 Laden Mass (D4) less than, or equal to the Suspension Manufacturer's Load Rating (B8)? | ☐ | ☐ |
| 4 | Is the Front Axle Group S10 Laden Mass (D4) less than, or equal to the Total Axle Group Tyre Load Capacity (B12)? | ☐ | ☐ |
| 5 | Is the Front Axle Group S10 Laden Mass (D4) less than or equal to the Front Axle Group S10 Conditional Mass Limit (D7)? | ☐ | ☐ |
| 6 | Is the Rear Axle Group S10 Laden Mass (D4) less than or equal to the vehicle manufacturer’s Rear Axle Load Rating (B4)? | ☐ | ☐ |
| 7 | Is the Rear Axle Group S10 Laden Mass (D5) less than, or equal to the Axle Manufacturer's Load Rating (B7)? | ☐ | ☐ |
| 8 | Is the Rear Axle Group S10 Laden Mass (D5) less than, or equal to the Suspension Manufacturer's Load Rating (B9)? | ☐ | ☐ |
| 9 | Is the Rear Axle Group S10 Laden Mass (D5) less than, or equal to the Total Axle Group Tyre Load Capacity (B15)? | ☐ | ☐ |
| 10 | Is the Vehicle S10 Laden Mass (D6) less than or equal to the GVM rating (B1)? | ☐ | ☐ |
| 11 | Is the Vehicle S10 Laden Mass (D6) less than or equal to the Total Laden S10 Conditional Mass Limit (D8)? | ☐ | ☐ |

Part E – S10 Livestock Loading Modification Plate

Vehicle Manufacturer's Plate

| Vehicle Manufacturer's Plate Details |  |
| --- | --- |
| - Record information from the vehicle manufacturer’s rating plate, affixed to the cabin by the manufacturer. - Where the vehicle has been assessed/modified/re-rated and approved in accordance with VSB 6, use the information from the VSB 6 modification plate. |  |
| Manufacturer Make/Model: |  |
| Maximum Front Axle Group Rating: | kg |
| Front Axle Group Tyre Size: |  |
| Front Axle Group Tyre Capacity: | kg |
| Maximum Rear Axle Group Rating: | kg |
| Rear Axle Group Tyre Size: |  |
| Rear Axle Group Tyre Capacity: | kg |
| Tare Mass: | kg |
| Gross Vehicle Mass (GVM) Rating (B1): | kg |
| Gross Combination Mass (GCM) Rating (B2): | kg |

S10 Modification Plate Details

| S10 Livestock Loading Plate |  |
| --- | --- |
| - Record information below for the relevant fields of the S10 modification plate. - Duplicate the below information on the livestock loading S10 plate (maroon in colour). - Affix the plate to the vehicle in a location that is easily visible and accessible. - All unused fields on the Livestock loading (S10) Plate must be blanked out with three (3), or more, ‘X’ (For example: ‘XXX’) |  |
| Vehicle Identification Number (VIN)/Chassis Number: |  |
| Approved Vehicle Examiner Number: |  |
| Certificate Number: |  |
| Date: |  |
| Vehicle S10 Laden Mass: (D6) | kg |
| Front Axle Group S10 Laden Mass: (D4) | kg |
| Rear Axle Group S10 Laden Mass: (D5) | kg |
| Front Axle Group S10 Tyre Size/Load Rating: (B10/B11) |  |
| Rear Axle Group S10 Tyre Size/Load Rating: (B13/B14) |  |
| Total Vehicle Unladen Mass: (C3) | kg |

Part G – Authorisation & Declaration

AVE Authorisation

| Declaration by certifier (AVE) |  |  |
| --- | --- | --- |
| I am the Approved Vehicle Examiner who completed all the mass calculations and declare that the information in this form is true and correct. |  |  |
| Issued by: | Company (if applicable): | AVE no.: |
| Signed: | Telephone: | Date: |

Owner/supplier Authorisation

| As the ☐ Owner / ☐ Supplier (select applicable) of the vehicle described in this form, I declare that the vehicle specifications and vehicle equipment detailed herein are representative of the vehicle as it will enter into service and that I have enlisted the services of the AVE mentioned above to complete the S10 approval of this vehicle in this build state/configuration. |  |  |  |
| --- | --- | --- | --- |
| Name: | Company (if applicable): |  |  |
| Address: |  |  | Postcode: |
| Signed: | Telephone: | Date: |  |

Prime Mover - Sheet 1:- Prime Mover

Wheelbase

D

15,000kg Load on Fifth Wheel (MSemi)

Toolbox

DToolbox

100kg (MToolbox)

Rearward Axle Mass - RAM:

RAMSemi:

RAMSemi = ((B5 – D) ÷ B5) x MSemi

= ((_______ - _______) ÷ _______) x _______

= ________ kg

RAMToolbox:

RAMToolbox = ((B5 – DToolbox) ÷ B5) x MToolbox

= ((_______ - _______) ÷ _______) x _______

= ________ kg

RAMTotal:

(D3) RAMTotal = RAMLoad + RAMToolbox

= ________ + ________

= _________ kg

Forward Axle Mass - FAM:

FAMSemi:

FAMSemi = (D ÷ B5) x MSemi

= (_______ ÷ _______) x ________

= ________ kg

FAMToolbox:

FAMToolbox = (DToolbox ÷ B5) x MToolbox

= (_______ ÷ _______) x ________

= ________ kg

FAMTotal:

(D2) FAMTotal = FAMSemi + FAMToolbox

= ________ + ________

= _________ kg

Fifth Wheel Offset:

(B17) Fifth Wheel Offset (D): _______ m

If D is further rearwards than the rear overhang line then D value is negative.

If D = 0m, (D2) FAM = 0kg and (D3) RAM = D1.

Centre of Mass to ROH Line - DToolbox:

DToolbox (m): (If applicable)

DToolbox = _______ m

Wheelbase:

(B5) Vehicle Wheelbase: _________ m

Notes:

Total S10 Laden Mass:

(C1) Front Axle Group Unladen Mass: _______ kg

(C2) Rear Axle Group Unladen Mass: _______ kg

Front Axle Group S10 Laden Mass:

(D4) Front Axle Group S10 Laden Mass = C1 + D2

= ______ + ______

= _______ kg

Rear Axle Group S10 Laden Mass:

(D5) Rear Axle Group S10 Laden Mass = C2 + D3

= ______ + ______

= ______ kg

Vehicle S10 Laden Mass:

(D6) Vehicle S10 Laden Mass = D4 + D5 = ______ + ______

= ______ kg

Prime Mover - Sheet 1:- Prime Mover